MeteoStation

Technical Documentation & User Manual

Version 1.0

Embedded weather station on M5Stack CoreS3 SE

ESP32-S3 · MicroPython · ENV III · Open-Meteo API

# Table of Contents

# 1. Project Overview

MeteoStation is a standalone embedded weather station built on the M5Stack CoreS3 SE platform. The device measures indoor temperature, humidity, and atmospheric pressure through an ENV III sensor, retrieves outdoor weather data from the Open-Meteo service over WiFi, and presents all information on a touch-enabled color display across five navigable screens.

The project is written in pure MicroPython without block-based environments, giving full control over rendering, memory management, and hardware resources. The interface follows a glacier/Garmin design language — a dark icy palette with accents, faux-depth effects, and subtle animation.

## 1.1 Key Features

- Multi-screen interface — five screens: main, data, weather, forecast, settings, navigated via a bottom bar

- Local measurements — temperature, humidity, pressure from the ENV III sensor every 5 seconds

- Outdoor weather — current conditions, forecast by parts of day, tomorrow's outlook from the Open-Meteo API

- Weather caching — the latest data is stored on flash and shown immediately on power-up, before WiFi connects

- Derived metrics — heat index (feels-like temperature), altitude above sea level, atmospheric pressure trend

- Power saving — automatic sleep mode with wake-on-touch

- Time synchronization — NTP at boot and every 24 hours

- Persistent settings — brightness, sleep timer, units, and weather refresh interval are stored on flash

## 1.2 Hardware Platform

| Component | Specification |
| --- | --- |
| Controller | M5Stack CoreS3 SE (ESP32-S3) |
| Display | 2.0" IPS, 320×240, ILI9342C, capacitive touch |
| Sensor | ENV III — SHT30 (temp/humidity) + QMP6988 (pressure) |
| Sensor connection | Grove Port A, I2C: SCL=GPIO1, SDA=GPIO2 |
| RGB LED | WS2812C on GPIO5, driven via RMT |
| Power | M5GO Battery Bottom 3 (500 mAh) |
| Memory | ESP32-S3 with PSRAM, 16 MB flash (LittleFS) |
| RTC | BM8563, battery-backed |

## 1.3 Software Stack

| Layer | Technology |
| --- | --- |
| Language | MicroPython (UIFlow2 firmware) |
| Graphics | M5GFX via the M5.Lcd canvas API |
| HTTP | requests (built into the firmware) |
| Weather API | Open-Meteo (current + hourly + daily) |
| File system | LittleFS on flash |
| Icon format | PNG with alpha channel, rendered via drawImage |

# 2. System Architecture

The application is built around a single non-blocking event loop that coordinates sensor polling, touch handling, background weather updates, and screen redraws. Each subsystem has its own timer based on `time.ticks_ms()`, allowing different tasks to run at different intervals without blocking one another.

## 2.1 File Structure

| File | Purpose |
| --- | --- |
| main.py | Core logic: screens, event loop, sensors, weather, settings |
| wcache.py | Weather cache module on flash (save/load/age) |
| icomod.py | Weather-icon module: WMO code mapping and PNG loading |
| config.json | Persistent user settings |
| /flash/icons/ | Weather PNG icons (18 files) |
| /flash/icons/ui/ | UI icons: nav bar, cards, status |
| /flash/cache/ | Cached weather.json file |

## 2.2 Non-Blocking Event Loop

Instead of delays via `time.sleep()` that would freeze the whole device, the loop spins roughly 20 times per second and checks each timer independently. The `needs_redraw` flag ensures the screen is repainted only on real data changes, conserving resources.

while True:

M5.update()

now = time.ticks_ms()

needs_redraw = False

# Touch — checked every iteration

if M5.Touch.getCount() > 0:

tx, ty = M5.Touch.getX(), M5.Touch.getY()

# ... navigation, wake, settings

# Sensors — every 5 seconds

if time.ticks_diff(now, last_sensor_ms) >= SENSOR_MS:

last_sensor_ms = now

t, h, p = read_sensors(sensor)

sensor_data.update(temp=t, humi=h, press=p, ...)

needs_redraw = True

# Weather — every 15/30/60 min (from config)

if time.ticks_diff(now, last_weather_ms) >= weather_ms:

# ... connect_wifi -> fetch_weather -> disconnect

if needs_redraw and not is_sleeping:

render()

time.sleep_ms(50)

## 2.3 Task Intervals

| Task | Interval | Constant |
| --- | --- | --- |
| Sensor polling | 5 seconds | SENSOR_MS |
| History graph point | 1 minute | GRAPH_MS |
| Weather refresh | 15/30/60 min | weather_refresh_min |
| NTP synchronization | 24 hours | NTP_MS |
| Topbar refresh (minute) | 1 minute | — |

## 2.4 Boot Sequence

The initialization order is carefully designed to avoid a white screen flash and to show useful data as fast as possible:

- init_hardware() — M5.begin() then immediately setBrightness(0), screen dark

- show_splash() — splash screen drawn in darkness

- fade_in() — smooth brightness ramp over 300ms

- rgb_pulse_animation() — RGB LED greeting

- read_sensors() — first read of local data

- wcache.load() — load cached weather from flash

- render() — first paint: sensors + cached weather

- do_wifi_tasks() — WiFi, NTP, fresh weather

- import icomod — icon module loaded AFTER WiFi

- render() — final update with fresh data

Loading the icon module after WiFi is a deliberate choice: during network connection the internal SRAM is occupied by the network stack, so deferring PNG sprite loading prevents a memory conflict.

# 3. Interface Screens

The interface consists of five screens. Switching happens by touching the corresponding zone of the bottom navigation bar. The active screen is highlighted with a frame around its icon.

## 3.1 Main Screen

An overview of local measurements in four cards: temperature (large figure with a thermometer icon), battery (with a dynamic color charge indicator), humidity, and atmospheric pressure. This is the default screen at power-up.

- Temperature — large value in °C with a thermometer icon

- Battery — charge percentage with color fill: green >50%, yellow >20%, red ≤20%

- Humidity — relative humidity in percent

- Pressure — atmospheric pressure in hPa

## 3.2 Data Screen

Extended metrics and derived calculations: feels-like temperature (heat index), altitude above sea level based on the barometric formula, and a pressure trend with an indicator icon (rising / stable / falling).

## 3.3 Weather Screen

Outdoor conditions from Open-Meteo: temperature, condition description, humidity, pressure, a large weather icon matching the WMO code, plus a comparison of indoor and outdoor temperature.

## 3.4 Forecast Screen

A forecast for the next three meaningful parts of the day (morning, day, evening, night), plus a tomorrow summary with maximum and minimum temperature, precipitation probability, and wind speed.

## 3.5 Settings Screen

Device configuration with immediate persistence to flash. Each touch on a row changes the corresponding parameter and writes config.json.

| Parameter | Values | Description |
| --- | --- | --- |
| Brightness | 40 / 120 / 220 | Display backlight brightness |
| Sleep timer | 1 / 2 / 5 / Off min | Time until automatic sleep |
| Temp unit | C / F | Temperature units |
| Weather refresh | 15 / 30 / 60 min | Weather update interval |
| Sea level hPa | ± 0.5 steps | Calibration for altitude calculation |
| NTP Sync | action | Manual time synchronization |

# 4. Key Subsystems

## 4.1 Weather Caching (wcache.py)

The module implements the stale-while-revalidate pattern: show stale data immediately, refresh in the background. This is the same technique browsers and mobile apps use for offline mode. Instead of an empty screen at boot, the user immediately sees the last known weather.

Data is stored in `/flash/cache/weather.json` as JSON with a save timestamp. At boot the module reads the file before WiFi connects, and after each successful fetch it overwrites it with fresh data.

def save(outdoor_weather_dict):

_ensure_dir()

payload = {

"saved_at": time.time(),   # UTC timestamp

"data":     outdoor_weather_dict,

}

with open(CACHE_FILE, 'w') as f:

json.dump(payload, f)

def load():

with open(CACHE_FILE, 'r') as f:

payload = json.load(f)

return payload.get("data"), payload.get("saved_at", 0)

## 4.2 Derived Metrics

### Feels-Like Temperature (Heat Index)

The Rothfusz formula approximates how temperature feels given humidity. It applies only when temperature ≥ 27°C and humidity ≥ 40%; outside these conditions the real temperature is returned.

### Altitude Above Sea Level

The barometric formula computes altitude from atmospheric pressure. It requires sea_level_hpa calibration via settings — this is not the current pressure but the pressure reduced to sea level.

def altitude_from_pressure(pressure_hpa):

sea = cfg.get('sea_level_hpa', 1013.25)

return 44330.0 * (1.0 - (pressure_hpa / sea) ** (1.0 / 5.255))

### Pressure Trend

Compares the current pressure with the value from 15 minutes ago. A threshold of ±0.5 hPa over 15 minutes matches the meteorological definition of a significant change. The result selects one of three icons: rising, stable, falling.

## 4.3 Icon System

All icons are stored as PNG with a transparent background and rendered via `canvas.drawImage(path, x, y)`. A key discovery during development: when drawImage paints a PNG onto a canvas (rather than directly to the display), M5GFX performs correct alpha compositing — transparent pixels reveal whatever is already in the canvas buffer. This allows icons to overlay any background without artifacts.

Icons are drawn BEFORE calling canvas.push(), making them part of a single atomic frame. This eliminates the flicker that would occur with two-phase rendering (background first, icon on top second).

| Category | Size | Purpose |
| --- | --- | --- |
| Weather | 64×64 | 18 weather-condition icons (WMO codes) |
| Cards | 48–64 | Thermometer, humidity, pressure, altitude, battery |
| Nav bar | 24×24 | 5 navigation icons |
| Trend | 24×24 | Rising / stable / falling pressure |
| Status | 16×16 | WiFi, refresh |

## 4.4 Sleep and Wake

After a period without touches (configurable: 1/2/5 min or off) the device sets brightness to 0, entering sleep mode. Any touch on the screen wakes the device, restores brightness, and repaints the current screen. This extends battery runtime.

# 5. Open-Meteo Integration

The device uses the free Open-Meteo API with no key required. A single HTTP request returns current weather, a 2-day hourly forecast, and a daily summary.

## 5.1 Request Structure

https://api.open-meteo.com/v1/forecast

?latitude=XX.XX&longitude=YY.YY

&current=temperature_2m,relative_humidity_2m,

surface_pressure,weather_code

&hourly=temperature_2m,weather_code,

wind_speed_10m,precipitation_probability

&daily=temperature_2m_max,temperature_2m_min,

precipitation_probability_max,wind_speed_10m_max

&forecast_days=2

&timezone=Europe/Your_Timezone

## 5.2 WMO Weather Codes

The API returns numeric WMO weather codes, which are mapped to text descriptions and matching icons through the `wmo_to_icon()` function in icomod.py.

| Code | Condition | Icon |
| --- | --- | --- |
| 0–1 | Clear / mostly clear | clear_day / clear_night |
| 2 | Partly cloudy | partly_cloudy |
| 3 | Overcast | overcast |
| 45–48 | Fog | fog |
| 51–55 | Drizzle | drizzle |
| 61–65 | Rain | rain / heavy_rain |
| 71–77 | Snow | snow |
| 95–99 | Thunderstorm | thunderstorm |

# 6. User Manual

## 6.1 First-Time Setup

- Connect the ENV III sensor to Grove Port A of the CoreS3 SE

- Remove the orange protective film before attaching the M5GO Battery Bottom

- Upload the project files to flash via Thonny: main.py, wcache.py, icomod.py and the icons/ folder

- Edit config.json — set wifi_ssid, wifi_pass and the city coordinates (weather_lat, weather_lon)

- Reboot the device — after the RGB animation the main screen appears

## 6.2 Navigation

The bottom bar contains five navigation zones. Touching a zone switches to the corresponding screen. The active screen is marked by a highlighted frame around its icon.

| Icon | Screen | Content |
| --- | --- | --- |
| Thermometer | Main | Temperature, battery, humidity, pressure |
| Graph | Data | Heat index, altitude, pressure trend |
| Sun | Weather | Outdoor weather from Open-Meteo |
| Calendar | Forecast | Forecast by parts of day and tomorrow |
| Wrench | Settings | Device settings |

## 6.3 Altitude Calibration

For correct altitude display, configure the Sea level hPa parameter in settings. This value is not the current pressure but the atmospheric pressure reduced to sea level. Adjust it with the ± 0.5 buttons until the altitude on the data screen matches the real elevation of your location.

## 6.4 Troubleshooting

| Symptom | Likely cause and fix |
| --- | --- |
| Empty screen (No weather) | WiFi not connected or no cache — check wifi_ssid/pass in config.json |
| Wrong time | NTP did not sync; check internet and utc_offset in config |
| Sensor reads 0.0 | Check ENV III connection to Grove Port A (I2C SCL=1, SDA=2) |
| Wrong altitude | Recalibrate Sea level hPa in settings |
| Icons not appearing | Verify files exist in /flash/icons/ and /flash/icons/ui/ |

# 7. config.json Reference

The `/flash/config.json` file holds all persistent settings. On first run it is created from defaults, then edited via the settings screen or manually.

| Key | Type | Default | Description |
| --- | --- | --- | --- |
| wifi_ssid | string | "" | WiFi network name |
| wifi_pass | string | "" | WiFi password |
| brightness | int | 120 | Brightness (40/120/220) |
| sleep_min | int | 2 | Minutes to sleep (0=off) |
| temp_unit | string | C | Temperature units |
| weather_lat | float | XX.XX | Latitude |
| weather_lon | float | YY.YY | Longitude |
| weather_city | string | Your City | City name in topbar |
| utc_offset | int | 2 | Offset from UTC (hours) |
| sea_level_hpa | float | 1013.25 | Altitude calibration |
| weather_refresh_min | int | 30 | Weather interval (min) |

# 8. Technical Notes and Limitations

Development revealed several characteristics of the M5Stack MicroPython platform worth keeping in mind for future work on the project.

## 8.1 M5GFX MicroPython Limitations

- The pushImage() method with raw RGB565 bytes is unavailable in the Python wrapper — only drawImage(path) from a file and push(x, y)

- push() takes only two arguments (x, y) and does not support color-key transparency

- Transparency solution: a PNG with an alpha channel via drawImage onto a canvas performs correct compositing

- Radial gradients are infeasible — 76800 pixels take seconds on the ESP32; linear bands are used instead

## 8.2 Python Scoping Caveat

A classic pitfall: reading a global variable inside a function works without declaration, but assignment creates a local variable. Every function that modifies `outdoor_weather`, `cfg`, or other global state requires an explicit `global` declaration.

## 8.3 Flash and Endurance

LittleFS has built-in wear leveling that evenly distributes writes across the whole flash. Even when writing sensor history every minute, the calculated endurance spans hundreds of years, so frequent cache writes are safe.

## 8.4 Roadmap

- Open-Meteo Archive API for 7-day weather history with real measurements

- Local database of sensor readings (binary circular buffer, ~1 MB)

- TrueType fonts instead of scaled bitmaps for crisper figures

- Transition animations on value changes

- Port to Arduino/C++ for full control over hardware resources

MeteoStation v1.0 — end of documentation